FVUI V0.12.28.7（202509301201）Version release log

- Introduction

- Firmware version: FVUI_7628_V0.12.28.7_202509301201_RC.bin

- Version upgrade：Upgrade from V0.12.12（202503192023）to V0.12.28.7（202509301201）

- Applicable models: P10/P10P/P10G/P10LTE/P11/P11P/P11G/P11LTE

- Release date: 2025.10.10

- New Features

- Diagnostic log adds monitoring application heartbeat function.

- Call prefix matching.

- Function optimization

- The default parameter for not verifying certificate validity has been added to both LDAPs and LDAP TLS start TLS connection methods.

- Add features to programmable buttons: Walkie-talkie, Interrupt, Group interrupt, Hold, Release hold, Private hold, Recording, URL recording, Voicemail, URL request, Mobile office.

- Ringing can be disabled during multicast.

- Problem fixing

- Fixed an issue where enabling automatic deployment of remote contacts for incoming/outgoing call matching did not take effect as intended. Users must manually click “Save” in the contacts settings for the changes to apply.

- Fixed an issue where, after a multicast receiver disconnects, the multicast sender initiates a new multicast too quickly, preventing the receiver from receiving the new multicast.

- Fixed issue where P10 imported incorrect configuration causing hotline calls to be placed when the handset was lifted.

- Fixed an issue where pressing the transfer key during a call did not respond in the customer environment.

- Fixed automatic deployment failure for AirSpring FIP12WP OPTION 66+ Metaswitch in the United States

- Fixed an issue where phones did not send requests to the link when updating the local address book using FACS remote address book.

- Fixed an issue where the remote phonebook download feature would cause the phone to ring and the screen to light up during scheduled updates.

- Fixed an issue where disabling key tones after using the inband DTMF mode failed to send DTMF signals.

- Fixed an issue where display names were incomplete when multiple display names existed in LDAP for incoming and outgoing calls.