Voicemail Blasting Configuration Guide

Directory

一、 Introduction	2

二、 Flow Configuration	2

1、 Configuring FreePBX	2

① Login to FreePBX administrator interface in browser	2

② Create an extension	2

③ Set Voicemail	3

④ Create a Voicemail Blasting Group	4

⑤ Check the Feature Codes of Voicemail	4

2、 Configuring IP Phone	4

① Login to the web interface of the IP phone	4

② Register an account	5

③ Set Voicemail	6

三、 Verification of Results	7

- Introduction

Voicemail blasting lets you send a voicemail message to multiple users at the same time. The Voicemail Blasting module is used to create a group of users and assign a number to the group. A user can dial this number to leave a voicemail message for the group. All members of the group will receive the message in their voicemail boxes.

- Flow Configuration

- Configuring FreePBX

- Login to FreePBX administrator interface in browser

Figure 1 FreePBX login screen

- Create an extension

Use PJSIP to create a SIP extension.

operating steps：

Applications--》Extensions--》Add Extensions--》Add New SIP [chan_pjsip]Extensions；

Figure 2 create an extension

Submit--》Apply Config；

- Set Voicemail

Enable the Voicemail function,and set it as the destination of the no answer.

operating steps：

Applications--》Extensions--》Add Extensions--》Add New SIP [chan_pjsip]Extensions--》	Voicemail；

Figure 3 set Voicemail function

- Create a Voicemail Blasting Group

Set VMBlast Number.

operating steps：

Applications--》Voicemail Blasting--》Add VM Blast Group；

Figure 4 set VMBlast

- Check the Feature Codes of Voicemail

Check the feature codes of voicemail，and dail the feature codes to listen to voicemails。

operating steps：

Admin--》Feature Codes--》Voicemail；

Figure 5 check feature codes

- Configuring IP Phone

- Login to the web interface of the IP phone

Enter the IP address of the IP phone in the browser, and enter the web interface of the 	phone for relevant configuration.

Figure 6 web interface

- Register an account

Fill in the extension information.

operating steps：

Enter the web interface of the 	phone--》VoIP--》Account--》Fill in the extension 	information--》save and reply；

Figure 7 register accounts

| Parameter | Description |
| --- | --- |
| Line Enable | Enable |
| Display Name | Fill in the display name |
| Phone Number | Fill in the extension number |
| Account | Fill in the account |
| Password | Fill in the password |
| Proxy Server | Fill in the ip address of FreePBX |
| Proxy Port | 5060（Check the server configuration for the actual port number） |

- Set Voicemail

Enable the voicemail function of ip phone，and fill in the feature codes。

操作步骤：

VoIP--》Account--》Supplementary Services；

Figure 8 enable voicemail function

| Parameter | Description |
| --- | --- |
| Enable MWI | Enable |
| MWI Subscribe Enable | Enable |
| Voice Mailbox Numbers | Fill in the feature codes of voicemail |

- Verification of Results

Users can dail the voicemail blasting group number: 1056 to send message to the voicemail box list.