FreePBX Docking Configuration Guide

Directory

一、 Introduction	2

二、 Flow Configuration	2

1、 Configuring FreePBX	2

① Login to FreePBX administrator interface in browser	2

② Create an extension	2

2、 Configuring IP Phone	3

① Login to the web interface of the IP phone	3

② Register an account	3

三、 Verification of Results	4

- Introduction

Register your phone with FreePBX and use it to make and receive calls.

- Flow Configuration

- Configuring FreePBX

- Login to FreePBX administrator interface in browser

Figure 1 FreePBX login screen

- Create an extension

Use PJSIP to create a SIP extension.

operating steps：

Applications--》Extensions--》Add Extensions--》Add New SIP [chan_pjsip]Extensions；

Figure 2 create an extension

Submit--》Apply Config；

- Configuring IP Phone

- Login to the web interface of the IP phone

Enter the IP address of the IP phone in the browser, and enter the web interface of the 	phone for relevant configuration.

Figure 3 web interface

- Register an account

Fill in the extension information.

operating steps：

Enter the web interface of the 	phone--》VoIP--》Account--》Fill in the extension 	information--》save and reply；

Figure 4 register accounts

| Parameter | Description |
| --- | --- |
| Line Enable | Enable |
| Display Name | Fill in the display name |
| Phone Number | Fill in the extension number |
| Account | Fill in the account |
| Password | Fill in the password |
| Proxy Server | Fill in the ip address of FreePBX |
| Proxy Port | 5060（Check the server configuration for the actual port number） |

- Verification of Results

- Register the extension numbers 1004 and 1122 on two phones respectively, use the extension 1004 to dial 1122, and the test is successful.

Figure 5 test result